Packet Tracer - Verifying IPv4 and IPv6 Addressing (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

Topology

- Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
|  |  | IPv6 Address/Prefix |  |  |
| R1 | G0/0 | 10.10.1.97 | 255.255.255.224 | N/A |
|  |  | 2001:DB8:1:1::1/64 |  | N/A |
|  | S0/0/1 | 10.10.1.6 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:1:2::2/64 |  | N/A |
|  | Link-local | FE80::1 |  | N/A |
| R2 | S0/0/0 | 10.10.1.5 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:1:2::1/64 |  | N/A |
|  | S0/0/1 | 10.10.1.9 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:1:3::1/64 |  | N/A |
|  | Link-local | FE80::2 |  | N/A |
| R3 | G0/0 | 10.10.1.17 | 255.255.255.240 | N/A |
|  |  | 2001:DB8:1:4::1/64 |  | N/A |
|  | S0/0/1 | 10.10.1.10 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:1:3::2/64 |  | N/A |
|  | Link-local | FE80::3 |  | N/A |
| PC1 | NIC | 10.10.1.100 | 255.255.255.224 | 10.10.1.97 |
|  |  | 2001:DB8:1:1::A/64 |  | FE80::1 |
| PC2 | NIC | 10.10.1.20 | 255.255.255.240 | 10.10.1.17 |
|  |  | 2001:DB8:1:4::A/64 |  | FE80::3 |

Objectives

Part 1: Complete the Addressing Table Documentation

Part 2: Test Connectivity Using Ping

Part 3: Discover the Path by Tracing the Route

Background

Dual-stack allows IPv4 and IPv6 to coexist on the same network. In this activity, you will investigate a dual-stack implementation including documenting the IPv4 and IPv6 configuration for end devices, testing connectivity for both IPv4 and IPv6 using ping, and tracing the path from end to end for IPv4 and IPv6.

Complete the Addressing Table Documentation

Use ipconfig to verify IPv4 addressing.

    - Click PC1 and click the Desktop tab > Command Prompt.

    - Enter the ipconfig /all command to collect the IPv4 information. Fill in the Addressing Table with the IPv4 address, subnet mask, and default gateway.

    - Click PC2 and click the Desktop tab > Command Prompt.

    - Enter the ipconfig /all command to collect the IPv4 information. Fill in the Addressing Table with the IPv4 address, subnet mask, and default gateway.

Use ipv6config to verify IPv6 addressing.

    - On PC1, enter the ipv6config /all command to collect the IPv6 information. Fill in the Addressing Table with the IPv6 address, subnet prefix, and default gateway.

    - On PC2, enter the ipv6config /all command to collect the IPv6 information. Fill in the Addressing Table with the IPv6 address, subnet prefix, and default gateway.

Test Connectivity Using Ping

Use ping to verify IPv4 connectivity.

    - From PC1, ping the IPv4 address for PC2. Was the result successful? Yes

    - From PC2, ping the IPv4 address for PC1. Was the result successful? Yes

Use ping to verify IPv6 connectivity.

    - From PC1, ping the IPv6 address for PC2. Was the result successful? Yes

    - From PC2, ping the IPv6 address of PC1. Was the result successful? Yes

Discover the Path by Tracing the Route

Use tracert to discover the IPv4 path.

    - From PC1, trace the route to PC2.

PC> tracert 10.10.1.20

What addresses were encountered along the path? 10.10.1.97, 10.10.1.5, 10.10.1.10, 10.10.1.20

With which interfaces are the four addresses associated? G0/0 of R1, S0/0/0 on R2, S0/0/01 on R3, NIC of PC2

    - From PC2, trace the route to PC1.

What addresses were encountered along the path? 10.10.1.17, 10.10.1.9, 10.10.1.6, 10.10.1.100

With which interfaces are the four addresses associated? G0/0 of R3, S0/0/1 of R2, S0/0/1 of R1, NIC of PC1

Use tracert to discover the IPv6 path.

    - From PC1, trace the route to the IPv6 address for PC2.

PC> tracert 2001:DB8:1:4::A

What addresses were encountered along the path? 2001:DB8:1:1::1, 2001:DB8:1:2::1, 2001:DB8:1:3::2, 2001:DB8:1:4::A

With which interfaces are the four addresses associated? g0/0 of R1, S0/0/0 of r2, S0/0/1 of R3, NIC of PC2

    - From PC2, trace the route to the IPv6 address for PC1.

What addresses were encountered along the path? 2001:DB8:1:4::1, 2001:DB8:1:3::1, 2001:DB8:1:2::2, 2001:DB8:1:1::A

With which interfaces are the four addresses associated? Ga0/0 of R3, S0/0/1 of R2, S0/0/1 of R1, NIC of PC1

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Complete the Addressing Table Documentation | Step 1b | 10 |  |
|  | Step 1d | 10 |  |
|  | Step 2a | 10 |  |
|  | Step 2b | 10 |  |
| Part 1 Total |  | 40 |  |
| Part 2: Test Connectivity Using Ping | Step 1a | 7 |  |
|  | Step 1b | 7 |  |
|  | Step 2a | 7 |  |
|  | Step 2b | 7 |  |
| Part 2 Total |  | 28 |  |
| Part 3: Discover the Path by Tracing the Route | Step 1a | 8 |  |
|  | Step 1b | 8 |  |
|  | Step 2a | 8 |  |
|  | Step 2b | 8 |  |
| Part 3 Total |  | 32 |  |
| Total Score |  | 100 |  |