# Activity 6.7.4: IPv4 Address Subnetting Part 2

## Learning Objectives

Upon completion of this activity, you will be able to determine subnet information for a given IP address and subnetwork mask.

## Background

Borrowing Bits

How many bits must be borrowed to create a certain number of subnets or a certain number of hosts per subnet?

Using this chart, it is easy to determine the number of bits that must be borrowed.

Things to remember:

- Subtract 2 for the usable number of hosts per subnet, one for the subnet address and one for the broadcast address of the subnet.

| 210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,024 | 512 | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| Number of bits borrowed: |  |  |  |  |  |  |  |  |  |  |
| 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 1 |
| 1,024 | 512 | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| Hosts or Subnets |  |  |  |  |  |  |  |  |  |  |

Possible Subnet Mask Values

Because subnet masks must be contiguous 1’s followed by contiguous 0’s, the converted dotted decimal notation can contain one of a certain number of values:

| Dec. | Binary |
| --- | --- |
| 255 | 11111111 |
| 254 | 11111110 |
| 252 | 11111100 |
| 248 | 11111000 |
| 240 | 11110000 |
| 224 | 11100000 |
| 192 | 11000000 |
| 128 | 10000000 |
| 0 | 00000000 |

## Scenario

When given an IP address, network mask, and subnetwork mask, you will be able to determine other information about the IP address such as:

- The subnet address of this subnet

- The broadcast address of this subnet

- The range of host addresses for this subnet

- The maximum number of subnets for this subnet mask

- The number of hosts for each subnet

- The number of subnet bits

- The number of this subnet

Task 1: For a Given IP Address and Subnet Mask, Determine Subnet Information.

Given:

| Host IP Address | 172.25.114.250 |
| --- | --- |
| Network Mask | 255.255.0.0 (/16) |
| Subnet Mask | 255.255.255.192 (/26) |

Find:

| Number of Subnet Bits |  |
| --- | --- |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Step 1: Translate host IP address and subnet mask into binary notation.

| IP Address | 172 | 25 | 114 | 250 |
| --- | --- | --- | --- | --- |
|  | 10101100 | 11001000 | 01110010 | 11111010 |
| Subnet Mask | 11111111 | 11111111 | 11111111 | 11000000 |
|  | 255 | 255 | 255 | 192 |

Step 2: Determine the network (or subnet) where this host address belongs.

- Draw a line under the mask.

- Perform a bit-wise AND operation on the IP Address and the Subnet Mask.

Note: 1 AND 1 results in a 1’ 0 AND anything results in a 0.

- Express the result in dotted decimal notation.

- The result is the Subnet Address of this Subnet, which is 172.25.114.192

|  | 172 | 25 | 114 | 250 |
| --- | --- | --- | --- | --- |
| IP Address | 10101100 | 11001000 | 01110010 | 11111010 |
| Subnet Mask | 11111111 | 11111111 | 11111111 | 11000000 |
| Subnet Address | 10101100 | 11001000 | 01110010 | 11000000 |
|  | 172 | 25 | 114 | 192 |

Add this information to the table:

| Subnet Address for this IP Address | 172.25.114.192 |
| --- | --- |

Step 3: Determine which bits in the address contain network information and which contain host information.

- Draw the Major Divide (M.D.) as a wavy line where the 1s in the major network mask end (also the mask if there was no subnetting). In our example, the major network mask is 255.255.0.0, or the first 16 left-most bits.

- Draw the Subnet Divide (S.D.) as a straight line where the 1s in the given subnet mask end. The network information ends where the 1s in the mask end.

- The result is the Number of Subnet Bits, which can be determined by simply counting the number of bits between the M.D. and S.D., which in this case is 10 bits.

Step 4: Determine the bit ranges for subnets and hosts.

- Label the subnet counting range between the M.D. and the S.D. This range contains the bits that are being incremented to create the subnet numbers or addresses.

- Label the host counting range between the S.D. and the last bits at the end on the right. This range contains the bits that are being incremented to create the host numbers or addresses.

Step 5: Determine the range of host addresses available on this subnet and the broadcast address on this subnet.

- Copy down all of the network/subnet bits of the network address (that is, all bits before the S.D.).

- In the host portion (to the right of the S.D.), make the host bits all 0s except for the right-most bit (or least significant bit), which you make a 1. This gives us the first host IP address on this subnet, which is the first part of the result for Range of Host Addresses for This Subnet, which in the example is 172.25.114.193.

- Next, in the host portion (to the right of the S.D.), make the host bits all 1s except for the right-most bit (or least significant bit), which you make a 0. This gives us the last host IP address on this subnet, which is the last part of the result for Range of Host Addresses for This Subnet, which in the example is 172.25.114.254.

- In the host portion (to the right of the S.D.), make the host bits all 1s. This gives us the broadcast IP address on this subnet. This is the result for Broadcast Address of This Subnet, which in the example is 172.25.114.255.

Let’s add some of this information to our table:

| Host IP Address | 172.25.114.250 |
| --- | --- |
| Major Network Mask | 255.255.0.0 (/16) |
| Major (Base) Network Address | 172.25.0.0 |
| Major Network Broadcast Address | 172.25.255.255 |
| Total Number of Host Bits Number of Hosts | 16 bits or 216 or 65,536 total hosts 65,536 – 2 = 65,534 usable hosts |
| Subnet Mask | 255.255.255.192 (/26) |
| Number of Subnet Bits Number of Subnets |  |
| Number of Host Bits per Subnet Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Step 6: Determine the number of subnets.

The number of subnets is determined by how many bits are in the subnet counting range (in this example, 10 bits).

Use the formula 2n, where n is the number of bits in the subnet counting range.

- 210  = 1024

| Number of Subnet Bits Number of Subnets (all 0s used, all 1s not used) | 10 bits 210 = 1024 subnets |
| --- | --- |

Step 7: Determine the number usable hosts per subnet.

The number of hosts per subnet is determined by the number of host bits (in this example, 6 bits) minus 2 (1 for the subnet address and 1 for the broadcast address of the subnet).

26 – 2 = 64 – 2 = 62 hosts per subnet

| Number of Host Bits per Subnet Number of Usable Hosts per Subnet | 6 bits 26 – 2 = 64 – 2 = 62 hosts per subnet |
| --- | --- |

Step 8: Final Answers

| Host IP Address | 172.25.114.250 |
| --- | --- |
| Subnet Mask | 255.255.255.192 (/26) |
| Number of Subnet Bits Number of Subnets | 10 bits 210 = 1024 subnets |
| Number of Host Bits per Subnet Number of Usable Hosts per Subnet | 6 bits 26 – 2 = 64 – 2 = 62 hosts per subnet |
| Subnet Address for this IP Address | 172.25.114.192 |
| IP Address of First Host on this Subnet | 172.25.114.193 |
| IP Address of Last Host on this Subnet | 172.25.114.254 |
| Broadcast Address for this Subnet | 172.25.114.255 |

Task 2: Challenge.

For all problems:

Create a Subnetting Worksheet to show and record all work for each problem.

Problem 1

| Host IP Address | 172.30.1.33 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Problem 2

| Host IP Address | 172.30.1.33 |
| --- | --- |
| Subnet Mask | 255.255.255.252 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Problem 3

| Host IP Address | 192.192.10.234 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Problem 4

| Host IP Address | 172.17.99.71 |
| --- | --- |
| Subnet Mask | 255.255.0.0 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Problem 5

| Host IP Address | 192.168.3.219 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Problem 6

| Host IP Address | 192.168.3.219 |
| --- | --- |
| Subnet Mask | 255.255.255.252 |
| Number of Subnet Bits |  |
| Number of Subnets |  |
| Number of Host Bits per Subnet |  |
| Number of Usable Hosts per Subnet |  |
| Subnet Address for this IP Address |  |
| IP Address of First Host on this Subnet |  |
| IP Address of Last Host on this Subnet |  |
| Broadcast Address for this Subnet |  |

Task 3: Clean Up

Remove anything that was brought into the lab, and leave the room ready for the next class.