# 10.7.1: Skills Integration Challenge: Network Planning and Interface Configuration

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| R2 | Fa0/0 |  |  | N/A |
|  | Fa0/1 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| R3 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| PC-1A | NIC |  |  |  |
| PC-2A | NIC |  |  |  |
| PC-3A | NIC |  |  |  |
| Eagle_Server | NIC |  |  |  |

## Learning Objectives

Upon completion of this lab, you will be able to:

- Build the network topology.

- Plan the IP addresses.

- Configure router and PC interfaces.

- Test the network.

## Background

Practice your network building, planning, and configuration skills. Device names and routing have already been configured.

## Task 1: Build the Network Topology.

Use the following charts, and the devices in the Device Pool, to create the topology.

Routers:

| Hostname | Interface | Connects To | Interface |
| --- | --- | --- | --- |
| R1 | Fa0/0 | SW-1 | Fa0/1 |
| R1 | S0/0/0 (DCE) | R2 | S0/0/0 |
| R1 | S0/0/1 (DCE) | R3 | S0/0/1 |
| R2 | Fa0/0 | SW-2A | Fa0/1 |
| R2 | S0/0/1 (DCE) | R3 | S0/0/0 |
| R2 | Fa0/1 | SW-2B | Fa0/1 |
| R3 | Fa0/0 | SW-3 | Fa0/1 |

Switches:

| Hostname | Interface | Connects To | Interface |
| --- | --- | --- | --- |
| SW-1 | Fa0/2 | PC-1A | FastEthernet |
| SW-2A | Fa0/2 | PC-1B | FastEthernet |
| SW-2B | Fa0/2 | Eagle_Server | FastEthernet |
| SW-3 | Fa0/2 | PC-1C | FastEthernet |

## Task 2: Create and Assign an Addressing Scheme.

You are asked to use the 192.168.1.0 /24 address space. Seven total networks are required; assign the networks in decreasing order of number of hosts required for efficient use of address space. Use the following charts to create an effective addressing scheme:

LAN:

| Hostname | Interface | Number of Hosts |
| --- | --- | --- |
| R1 | Fa0/0 | 60 |
| R2 | Fa0/0 | 10 |
|  | Fa0/1 | 30 |
| R3 | Fa0/0 | 7 |

WAN:

| Hostname | Address to be Assigned | Number of Hosts |
| --- | --- | --- |
| R1-R2 | R1-First host address | 2 |
| R1-R3 | R1-First host address | 2 |
| R2-R3 | R2-First host address | 2 |

Use the following rules to assign the IP addresses.

- PCs will use the first host address in the subnet; the server will use the second to last host address in its subnet.

- All FastEthernet ports on a router will use the last host address of the assigned subnet.

- The R1-R2 link will use the first WAN subnet, the R1-R3 link will use the second WAN subnet, and the R2-R3 link will use the third WAN subnet. R1 and R2 DCE interfaces should have clock rates of 56000.

## Task 3: Interface Configuration

Perform interface configuration of the R1, R2, and R3 routers, the PCs, and the server according to the addressing scheme above.

## Task 4: Testing Connectivity

Make sure all PCs can ping their gateways, other PCs, and the server.